30W International Standard PD

802.3AF/AT     12V 2A  Isolated Type

一、Feature：

- ND-PD3824 is a high performance POE module with photoelectric isolation,comply with IEEE 802.3AF/AT standard, the size is 38*38*15mm.The output power can meet more than 99% of the market demand for camera power consumption.

- •ND-PD3824 can be easily matched with more than 99% of the modules on the market, such as XM,  HIKVISION, Dahua and many other IPC module manufacturers.

• Adopt high-end POE chip design, with isolation transformer and a variety of protection circuits.

• POE protocol: IEEE802.3af/at

• POE Power: Full 24W, continuous work for  long time,very reliable power ,DC(OUT):12V/2A, 4-layer PCB design, continuous low operating temperature

• Isolation voltage: 1500V

• POE transmission: 1,2+/3,6- and 4,5+/7,8- adaptive

二、Electrical parameter ：

| Voltage Typr | PoE switch input 48V |  |  |
| --- | --- | --- | --- |
| Paramater | minimum | Rated Value | Maximum |
| input | 42Vdc | 48Vdc | 56Vdc |
| output | 11.5V | 12V | 12.5V |
| Input current | 0.03A | 2A | 2.4A |
| mVpk=pk | 200mV Max |  |  |
| Efficiency | Beyond 80% |  |  |
| Short-circuit protection | When the output is short-circuited to the ground, the input power of the product is reduced and will not be damaged. When the short-circuit situation is removed, the product will automatically return to normal |  |  |

三、PIN defined description:

| PIN | Definition |  | remarks |
| --- | --- | --- | --- |
| J3 | 1 | DC+ | PoE switch input 45V |
|  | 2 | DC- | PoE Switch input 78V |
|  | 3 | GND | 12V output |
|  | 4 | +12V |  |
| J2 | 1 | GND | 12V output |
|  | 2 | +12V |  |
|  | 3 | TX+ | switch data output P1 |
|  | 4 | TX- | switch data output P2 |
|  | 5 | RX+ | switch data output P3 |
|  | 6 | RX- | switch data output P6 |
|  | 7 | LED+ | External indicator light is connected to entrance |
|  | 8 | LED- |  |
| J1 | 1 | LED- | Camera indicator light signal port |
|  | 2 | LED+ |  |
|  | 3 | RX- | switch data input P6 |
|  | 4 | RX+ | switch data input P3 |
|  | 5 | TX- | switch data input P2 |
|  | 6 | TX+ | switch data input P1 |

四、Environmental/humidity requirements：

Operating temperature and humidity requirements

For altitudes less than or equal to 10,000 feet, low temperature operating limit is -25 ° C, high temperature operating limit is +85 ° C, and relative humidity is 20%RH to 80%RH.

• Storage temperature and humidity requirements

Low temperature storage minimum of -30 ° C (ice-free environment) at altitudes below 30,000 ft.The upper limit of high temperature storage is +80℃, and the relative humidity is 10%RH ~ 90%RH